MUSIC MAN CONSOLE

Claude Code Handoff Document

Broken Arrow Expedition · Adventure Guides

Boring Badger (Chris) · Gaming Gecko (Henry) · Little Red Machine (Nolan)

# 1. What This Is

A self-contained, battery-powered, wireless show production system for Adventure Guides campfire ceremonies. Built on a Raspberry Pi 4B with a Flask backend, Stream Deck MK.2 physical controller, iPad Mini operator display, and a full WiFi lighting system. Designed to be operated by a volunteer dad or even a kid, with zero technical knowledge required after setup.

The system runs entirely offline — it creates its own WiFi network at camp. Everything connects to it: the iPad, the MC remote, the WLED light strips, the moving head spotlights. The operator interface is a web page served by the Pi.

# 2. Current State of the Pi

## SSH Access

• Hostname: musicman.local

• User: pi

• Service: musicman.service (systemd, auto-start on boot)

• App directory: /home/pi/musicman/

## Directory Structure

/home/pi/musicman/

musicman.py         ← Main Flask app (NEEDS REWRITE)

config.yaml         ← System config (working, has data)

venv/               ← Python virtual environment

static/             ← HTML files served by Flask

musicman_ui.html  ← Operator console (needs wiring)

musicman_admin.html ← Admin panel (needs rewrite)

updater.html      ← System file updater (working)

assets/             ← All media files

circles/circle_1/ ← circle_1 has logo.png uploaded

sfx/              ← Empty — no SFX files yet

music/            ← Empty

logs/               ← musicman.log, error.log

campouts/           ← Empty

## What's Working

• Flask service starts and stays running on port 80

• musicman.local resolves on the network

• Operator UI loads at http://musicman.local

• Admin panel loads at http://musicman.local/admin

• Updater page at http://musicman.local/update — can push new files without SSH

• config.yaml loads correctly — has 9 circles, Mighty Creek saved as circle_1

• /api/circles returns all 9 circles as JSON

• File uploads land in the correct asset folders

• config.yaml updates when assets are uploaded (fixed in latest musicman.py)

• Timer logic in musicman.py is correct

• Audio engine (pygame) initializes correctly

• Kill everything endpoint works

• WebSocket infrastructure is in place

## What's Broken / Not Built Yet

| ITEM | STATUS | NOTES |
| --- | --- | --- |
| Admin panel circle grid | ⚠️ Partial | Shows circles but has JS bugs around editCircle/save flow |
| Asset persistence on re-click | ⚠️ Partial | Files upload correctly but slot shows 'No file' when re-clicking |
| Save circle feedback | ⚠️ Partial | Says saved but grid refresh unreliable |
| Admin panel roles tab | 🔲 Not wired | UI exists, not connected to backend |
| Admin panel sounds upload | 🔲 Not wired | UI exists, backend endpoint exists, not tested |
| Stream Deck daemon | 🔲 Not built | No streamdeck.py file exists yet |
| WLED lighting calls | 🔲 Not tested | Code exists in musicman.py, no WLED devices yet |
| Art-Net DMX moving heads | 🔲 Not built | Architecture designed, no code |
| Animation generator | 🔲 Not built | generator.py referenced but doesn't exist |
| Display page (projector) | 🔲 Not built | display.html doesn't exist |
| MC remote firmware | 🔲 Not built | ESP32 code not written |
| Battery monitoring | 🔲 Not built | INA219 integration not written |
| Program runner / show flow | 🔲 Not built | Designed, not implemented |
| Campout setup wizard | 🔲 Not built | Designed, not implemented |
| Applause meter | 🔲 Not built | API endpoints exist, no UI |
| Call & response | 🔲 Not built | API endpoints exist, no UI |
| Playlist player | 🔲 Not built | Architecture designed |
| Pre-show checklist | 🔲 Not built | Designed, not implemented |

# 3. Immediate Priorities for Claude Code

Fix these in order — they are blocking all further progress:

## Priority 1: Rewrite musicman_admin.html from scratch

The current admin panel has accumulated too many patches and has JS bugs. Rewrite it cleanly. The backend API is working — the HTML just needs to call it correctly.

The admin panel must do these things reliably:

• Load circles from /api/circles on page load

• Display all 9 circles as clickable cards in a grid

• Click a circle card → populate the editor with that circle's saved data

• Show correct filenames in asset slots (logo, walkup music, animation) based on what's in config.yaml

• Upload logo/music/animation files → land in correct folder → config.yaml updates → slot shows filename

• Save circle name/color/walkup settings → config.yaml updates → grid refreshes → card shows new name

• All saves give clear success/failure feedback

• Roles tab works the same way as circles

• Sounds tab: drag-and-drop upload to sfx/, music/ folders — works and shows file list

## Priority 2: Verify musicman.py upload route

The upload route was recently fixed to write filenames back to config.yaml. Verify this is working by:

• Upload a logo for circle_1

• Check /api/circles — circle_1.assets.logo should be 'logo.png'

• Click circle_1 card — logo slot should show 'logo.png'

## Priority 3: Audio test

Once admin panel is working, upload airhorn.mp3 to sfx/ and verify it plays via /api/sfx/play?name=airhorn

# 4. Full System Architecture

## Hardware

• Raspberry Pi 4B — Flask + Gunicorn backend, audio engine, WLED API, Art-Net DMX

• Stream Deck MK.2 — 15 backlit LCD keys, USB, page-flip for 4 pages = 60 logical buttons

• iPad Mini — kiosk mode Safari, shows operator UI at musicman.local

• Netgear router — dedicated show WiFi, everything connects to it

• Behringer UMC22 equivalent — USB audio to Fender Passport PA (3.5mm to dual 1/4" into CH4+CH5)

• Anker 737 power bank — powers all case components via USB-C PD

• Apache case — self-contained, bulkhead panel with audio out, USB-A, RJ45, C14 AC inlet

• MC Remote — XIAO ESP32-S3, 4 buttons, LiPo, HTTP to Flask over WiFi

• Light towers x2 — 9ft stands, WS2815 12V pole strips (10m each, 3 segments), moving head DMX, PAR can DMX, power station

• EZ-Up strips — S3 left, S4 right (crowd wash), S5 ceiling (backstage work light)

• S6 box lid strip — operator work light, timer color feedback

• Raleno LED panels x2 — stage fill, set once per show, no integration

## Software Stack

• Flask + Gunicorn — backend, 2 workers, 4 threads, port 80

• pygame — audio engine, pre-loaded files, SFX on separate channel

• python-streamdeck (pip: streamdeck) — Stream Deck driver

• WLED HTTP API — addressable strip control

• sacn / Art-Net UDP — moving head DMX control via ESP32 nodes

• MoviePy + Pillow — walk-up animation auto-generator

• flask-sock — WebSocket for real-time state push to UI

• pyyaml — config.yaml read/write

• smbus2 — INA219 battery monitoring

## Flask API Endpoints (what exists in musicman.py)

GET  /                    → musicman_ui.html

GET  /admin               → musicman_admin.html

GET  /update              → updater.html

GET  /display             → display.html (not built yet)

GET  /api/state           → full system state JSON

GET  /api/circles         → all circles from config

GET  /api/roles           → all roles from config

GET  /api/sfx/list        → sfx list from config

GET  /api/audio/stop      → stop all audio

GET  /api/audio/pause     → pause

GET  /api/audio/resume    → resume

GET  /api/audio/fade      → fade out (?duration=)

GET  /api/audio/volume    → set volume (?v=0-100)

GET  /api/sfx/play        → play SFX (?name=id)

GET  /api/timer/start     → start timer

GET  /api/timer/pause     → pause timer

GET  /api/timer/reset     → reset timer

GET  /api/timer/toggle    → start or pause

GET  /api/lights/scene    → set scene (?name=)

GET  /api/lights/brightness → set brightness (?v=)

GET  /api/macro/walkup    → fire walk-up (?circle= or ?role=)

GET  /api/kill            → kill everything

GET  /api/callresponse/stage/on|off → call & response

GET  /api/callresponse/crowd/on|off

GET  /api/applause/up|down → applause meter

GET  /api/config          → get/POST config

POST /api/admin/upload    → upload asset file

POST /api/admin/circle    → save circle data

POST /api/admin/role      → save role data

POST /api/admin/generate  → trigger animation generation

GET  /api/system/status   → CPU/memory/uptime

POST /api/system/restart  → restart service

POST /api/system/update-file → updater endpoint

WS   /ws                  → WebSocket for real-time state

# 5. Config.yaml Structure

The config file is the single source of truth. Everything reads from and writes to it. Key sections:

system:        hostname, port, debug, audio_device

expedition:    name, tagline, logo, season, authors

audio:         volumes, fade durations, preload setting

timer:         duration (180s), warning_at (15s), sounds

display:       resolution, idle animation, fonts

animation:     style presets for auto-generator

circles:       9 entries, each has id, name, color, assets{logo,walkup_music,animation}, walkup{start_time,duration,fade_duration}, deck{page,slot}

roles:         7 entries (navigator, log_keeper, story_teller, firestarter, candy_man, wizard, circle_navigator)

sfx:           16 entries with id, name, file, color, deck

scenes:        8 lighting scenes with per-zone behavior

moving_heads:  2 DMX fixtures with universe, address, IP

wled_devices:  6 nodes with IPs and segment counts

battery:       monitoring thresholds and capacities

macros:        9 show macros with step sequences

stream_deck:   4 pages with key assignments

mc_remote:     4 button mappings

applause_meter: level settings, colors

call_response: zone assignments, colors

program_template: 16-item campfire show flow

playlist:      shuffle, loop, volume settings

paths:         all directory locations on Pi

Current data state: circle_1 has name='Mighty Creek', color='#008fff', logo='logo.png' (uploaded). Circles 2-9 have generic names. No SFX files uploaded yet.

# 6. Campfire Show Flow

The system is designed around a specific Adventure Guides campfire ceremony structure:

• Gathering / Pre-show — playlist music, campfire lighting

• Navigator entrance — walk-up music + spotlight

• Pledge of Allegiance — clean white lighting, respectful

• Log Keeper + Roll Call — calls each circle by name, circle comes up and does cheer, walk-up music + circle color lights fire

• Firestarter — dramatic two-stage sequence: music build → blackout → operator hits FIRE LIT when fire catches → campfire scene locks in

• Story Teller — intimate dim warm lighting

• Circle Navigator — explains skit stick, uses skit circle's color

• Skits x3 — walk-up + skit timer (3:00 countdown, warning at 0:15, airhorn at 0:00)

• Candy Man — crowd lights, party energy

• Wizard — eerie entrance, pin ceremony

• Awards — fanfare, party lights

• Closing + Send-off — fade out, campfire warm

Each role has configurable intensity: Simple (spotlight only), Standard (music + lights), Full Send (full theatrical sequence). Configured per-campout in setup wizard.

# 7. Lighting System

## WLED Devices (WiFi strips)

• tower_a — Tower A pole strips, IP 192.168.1.101, 2 segments: crowd (360 LEDs) + stage (240 LEDs)

• tower_b — Tower B pole strips, IP 192.168.1.102, same structure

• ezup_left — S3 EZ-Up left side, IP 192.168.1.103

• ezup_right — S4 EZ-Up right side, IP 192.168.1.104

• ezup_ceil — S5 EZ-Up ceiling, IP 192.168.1.105, independent (always on dim warm white)

• box_lid — S6 case lid, IP 192.168.1.106, timer feedback (amber→yellow→red)

All WS2815 12V strips. Not yet installed — IPs are planned addresses.

## DMX Devices (moving heads + PAR cans)

• Tower A: ESP32 Art-Net node at 192.168.1.201 → DMX → PAR can (addr 1) → Moving head (addr 5)

• Tower B: ESP32 Art-Net node at 192.168.1.202 → same structure

• Not yet built or purchased

## Lighting Scenes

• campfire — fire simulation, warm amber throughout

• spotlight — clean white stage wash

• pledge — clean white, all zones, respectful

• party — rainbow cycle

• storm — blue/white strobe

• spooky — purple/green twinkle

• blackout — all off (ceiling + box lid stay on for safety)

• campfire_lock — fire lit, locks in permanently

# 8. Stream Deck Layout

Stream Deck MK.2 — 15 keys, 4 pages:

• Page 1 — Walk-Up: slots 1-9 for circles, 10-11 for Navigator/Log Keeper, 12 timer start/pause, 13 timer reset, 14 STOP ALL, 15 → Page 2

• Page 2 — Sound FX: slots 1-14 for SFX, 15 → Page 3

• Page 3 — Lighting: scene buttons, dim +/-, 15 → Page 4

• Page 4 — Macros: show macros + staff roles, 14 STOP ALL, 15 → Page 1

Stream Deck daemon (streamdeck.py) is NOT built yet. This is a separate file that runs alongside Flask.

# 9. MC Remote

XIAO ESP32-S3 in a 3D printed enclosure. 4 buttons:

• Button 1 — Timer start/pause

• Button 2 — Airhorn

• Button 3 — Crowd up (applause meter)

• Button 4 — Crowd down (applause meter)

Connects to musicman WiFi network. Makes HTTP GET requests to Flask API. Firmware (mc_remote.ino) not written yet.

# 10. Key Design Decisions

• Single config.yaml as source of truth — never hardcode data in HTML or Python

• Idiot-proof operation — must work for a kid with no training

• No terminal required after initial setup — all files update via http://musicman.local/update

• Pre-load all audio to RAM at boot — instant SFX response, no disk seek

• WLED calls are concurrent (threading) — all strips change simultaneously

• Stream Deck calls Flask directly via function calls, not HTTP — zero latency

• Kill everything snaps to campfire warm, not blackout — safety in dark

• Timer fires warning sound AND visual (strip color change) at 15 seconds

• Walk-up macros: flash circle color → play music from trim point → auto-fade after duration → return to campfire scene

• config.yaml is human-readable YAML — can be edited in any text editor as last resort

• All assets stored in /home/pi/musicman/assets/ with predictable paths

• Updater page allows pushing new HTML/Python/config files without SSH

# 11. Recommended Build Order After Fixes

After admin panel is working and audio is tested:

1. streamdeck.py — Stream Deck daemon, key layout, live timer key

2. display.html — Projector fullscreen page, WebSocket listener, video playback

3. generator.py — MoviePy animation renderer, 7 style presets

4. mc_remote.ino — ESP32 firmware, 4 buttons, HTTP to Flask

5. Operator UI rebuild — show flow as primary, context panel, call & response, applause meter, kill button

6. Program runner — campout setup wizard, show flow with per-item triggers

7. Battery monitoring — INA219 integration, status in UI

8. Systemd hardening — install script fixes, production-ready service config

# 12. Pi Environment Notes

• OS: Raspberry Pi OS Lite 64-bit

• Python: 3.13.5 in venv at /home/pi/musicman/venv/

• Port 80: granted via setcap on /usr/bin/python3.13

• nginx: installed but disabled (was conflicting)

• Stream Deck udev rules: installed at /etc/udev/rules.d/70-streamdeck.rules

• Auto-reconnect rule: /etc/udev/rules.d/71-streamdeck-reconnect.rules

• Service file: /etc/systemd/system/musicman.service

• Gunicorn: 2 workers, 4 threads, gthread worker class

• WatchdogSec and NotifyAccess removed from service file (were causing crashes)

• pip package name: 'streamdeck' (not 'python-streamdeck')

• pip package name: 'hid' (not 'hidapi')

MUSIC MAN CONSOLE · BROKEN ARROW EXPEDITION · PALS FOREVER · ADVENTURE GUIDES

Boring Badger (Chris) · Gaming Gecko (Henry) · Little Red Machine (Nolan)